# Activity 2.

We evaluate a scenario with 4 servers. This model easily generalizes to a full data center setting where each server represents an entire rack or pod, simply by applying an appropriate scaling factor (e.g., 103for a server rack).

Open the dashboard and check the following default settings:

- Total workload capacity 180%

- Perfect load balancing

- Server: Idle Power Pidle=100 W, Max power Pmax=300 W

| Scenario | Server 1-4 Allocation % | Power (linear profile) | Power (onoff profile) | Power (cubic profile) |
| --- | --- | --- | --- | --- |
| S1 | 45,45,45,45 | 760 W | 1200 W | 473W |

The number of the active servers are 4.

Observe:

- the graphs of the POD Load Distribution and of the Total Datacenter Power are redundant with the other data reported in the dashboard;

- the final graph reports the three power profiles and shows the operating points of the four server (reported, for the sake of simplicity, just on the cubic curve);

- when modifying the server workload allocation, check that the "Total Datacenter Power" is the expected value;

- by construction, it is not possible to perform a direct comparison between the results obtained with different power profile models.

Q1: Consider an OnOff power profile. Find the power consumption for different workload.

| Workload Capacity Load | Power [W] for Consolidate | Power [W] for Consolidate with Load Balance | Power [W] for Load Balance |
| --- | --- | --- | --- |
| 80% |  |  |  |
| 160% |  |  |  |
| 240% |  |  |  |
| 320% |  |  |  |
| 400% |  |  |  |

Q2: What is the best policy for OnOff power profile? Is it the same for all possible workloads?

Q3: Consider a Linear power profile. Find the power consumption for different workload.

| Workload Capacity Load | Consolidate | Consolidate with Load Balance | Load Balance |
| --- | --- | --- | --- |
| 80% |  |  |  |
| 160% |  |  |  |
| 240% |  |  |  |
| 320% |  |  |  |
| 400% |  |  |  |

Q4: What is the best policy for linear power profile? Is it the same for all possible workloads?

Q5: Consider a Cubic power profile. Find the power consumption for different workload.

| Workload Capacity Load | Consolidate | Consolidate with Load Balance | Load Balance |
| --- | --- | --- | --- |
| 80% |  |  |  |
| 160% |  |  |  |
| 240% |  |  |  |
| 320% |  |  |  |
| 400% |  |  |  |

Q6: What is the best policy for cubic power profile? Is it the same for all possible workloads?

Q7: Comparing the results obtained with different power profiles, is it possible to identify the best policies in general?